For Dixon Brass part # 0010260C, 0010055C and 0010860C the crimp specs are as follows:

For a standard SAE 100R1 hose crimp to .455.

Dixon Brass grease hose assemblies are made with SGUN-03 grease whip hose.

CRIMP OD IS .455 +/- .005 (crimps usually measure between .451 to .459)

Maintain crimp length of approx. 5/8 (.625) to 11/16.

Measure down from the bottom of the hex nut 1/8 to 3/16 to start crimp length.